CH 245: ORGANIC CHEMISTRY 1 LABORATORY (Fall 2019)

Title:

- Purpose: (1 point)

The purpose of this lab is to perform a simple distillation and a vacuum distillation on a mixture of hexane and decane to separate the mixture. This process allows us to separate liquids in a mixture based on different intramolecular forces and to purify liquids in a solution.

- Drawing of structure of the main compound or balanced chemical equation if synthesis is performed: (1 point)

Hexane			Decane

3. Reagents and the major product (up to 6 points)

| Name | M.W. (0.5 pts) | Density (0.5 pts) | Amount (grams/mL) (0.5 pts) | Moles (0.5 pts) | Hazards/Precautions (MSDS data) and melting point or boiling point (2 pts) | Waste Disposal (aqueous or organic) (2 pts) |
| --- | --- | --- | --- | --- | --- | --- |
| Hexane | 86.18 g/mol | 0.659 g/cm3 |  |  | Eye and skin irritation. BP: 69°C | Organic |
| Decane | 142.29 g/mol | 0.73 g/cm3 |  |  | Dryness on skin or eye irritation. BP: 172-174°C | Organic |

4. Procedure (up to 2 points)

| Procedure | Observations and Lab Data |
| --- | --- |
| A summary of the procedure done with bullet points) | Color changes, exothermic or endothermic reactions, gas generation, etc.; tare weights for flasks, etc. |
| - Place a 20-mL sample of the unknown hexane and decane mixture into a tared, clean, dry 50mL round-bottomed flask. - Record the mass of the mixture. - Set up the apparatus as shown on the demo table, with tared receiver flask. - Start distillation at atmospheric pressure. - Record the head temperature at short time intervals after the liquid starts to come over and starts condensing into receiver flask. - When the temperature begins to fall (or rise suddenly) and no more condensate is being collected, replace receiver with a second, tared 20 mL receiver flask. - Remove heat and cool. - Attach vacuum line and gradually turn vacuum to maximum. Mixture must be at room temperature. - Resume distillation while recording temperature of head at intervals. - Continue distilling to dryness. - Turn off heat, allow the apparatus to cool, and turn off vacuum. |  |

5. Results; include actual yield in grams and % yield.

Weight of Hexane – 7.27 grams

Weight of Decane – 5.23 grams

|  | Hexane | Decane |
| --- | --- | --- |
| Weight | 7.27 grams | 5.23 grams |
| % Volume | 60.6% | 39.4% |
| % Mass | 58.2% | 41.8% |
| Mole fraction | 69.7% | 30.3% |

Volume of hexane = 7.27g * 1 cm3/0.659 cm3 = 11.03 mL

Volume of decane = 5.23g * 1 cm3/0.73 cm3 = 7.16 mL

Total volume = 18.19 mL

Moles of hexane = 7.27 grams / 86.18 g/mol = 0.0844 moles

Moles of decane = 5.23 grams / 142.29 g/mol = 0.0368 moles

Total moles = 0.121 moles

Boiling point of hexane – 68-69°C

Boiling point of decane – 116°C

I estimate the vacuum pulled by the house vacuum to be about 130 torr.

lnP2P1=ΔHvapR1T1-1T2
P2=P1*eΔHvapR1T1-1T2
P2=760*e465008.31451173+273-1116+273
P2=760*0.1592
P2=121 torr

Conclusion:

I accomplished a separation of decane and hexane in solution using two different types of distillation: simple distillation and vacuum distillation. I learned how to execute both methods of distillation, and learned not only about how two substances with a large enough difference in boiling point could be separated, but also about how useful a vacuum is when boiling substances to lower the amount of heat needed. One issue during the experiment was a lack of patience for the substance to cool enough before pulling the vacuum, resulting in a small amount of premature boiling, which may have caused some error. My future recommendation is to wait longer before pulling the vacuum, putting a thermometer in the solution to double check the temperature, or cooling the solution in an ice bath to speed up the process. The practical application of this lab is to separate two compounds with different boiling points, or to purify compounds in sensitive environments, such as drug making.